Global Procurement and Materials Department

Installation Guide

1.0

# OpenFOAM Setup Guide

Global Procurement and Materials Department – MPS Team

| Document Release Date: | 27 April 2026 |
| --- | --- |
| Prepared By: | ‘Affan Najiy Bin Rusdi |

Table of Contents

Change History	4

1.0 Introduction	5

Quick-Start Checklist	5

2.0 Windows Subsystem for Linux (WSL)	6

2.1 Installing WSL	6

2.2 Installing Ubuntu Linux Distribution	6

2.3 Opening WSL (Ubuntu Terminal)	6

Option A: Via the Windows Terminal UI	6

Option B: Via Command	6

3.0 Installing OpenFOAM	7

Step 1: Add the OpenFOAM Debian repository	7

Step 2: Update the package list	7

Step 3: Install OpenFOAM	7

4.0 Placing the Project Files	8

Step 1: Navigate to the root directory	8

Step 2: Create the directory	8

Step 3: Place the scripts	8

5.0 Installing Python Dependencies	8

6.0 Creating Bash Aliases	10

Step 1: Open the aliases file in vi	10

Step 2: Enter Insert mode	10

Step 3: Add the alias definitions	10

Step 4: Save and exit	10

Step 5: Apply the aliases to your current session	11

7.0 Launching OpenFOAM	11

Step 1: Source the OpenFOAM environment	11

Step 2: Navigate to the working directory	11

8.0 Running the Mesh Utilities	12

8.1 Graphical User Interface (Recommended)	12

8.2 Command-Line Scripts	12

Generate Background Mesh	12

Generate snappyHexMeshDict	12

9.0 Troubleshooting	13

# Change History

| Revision Changes |  |  |  |
| --- | --- | --- | --- |
| Rev. # | Last Updated | Author | Comments |
| 1.0 | 27 April, 2026 | ‘Affan Najiy Bin Rusdi | Initial Copy |

# PART 1 – USER GUIDE

# 1.0 Introduction

This document provides a step-by-step guide to installing Windows Subsystem for Linux (WSL),

Ubuntu, and OpenFOAM, along with the mesh utility scripts and aliases for daily use.

Compatible with ESI OpenFOAM v2506, Ubuntu 24.04 LTS, Windows 10/11

## Quick-Start Checklist

Use this checklist each time you start a new work session:

- Open Windows Terminal and select Ubuntu (or run: wsl -d Ubuntu).

- Source the OpenFOAM environment:  of2506

- Navigate to the project directory:  myDir

- Change into your specific case directory:  cd <case_name>

- Launch the GUI:  openfoamUI   — or run scripts directly.

Note: For persistent alias changes, always edit ~/.bash_aliases and then run source ~/.bash_aliases.

# 2.0 Windows Subsystem for Linux (WSL)

## 2.1 Installing WSL

Note: Skip this part if WSL and OpenFOAM are already installed.

Windows Subsystem for Linux (WSL) allows you to run a Linux environment directly on Windows without needing a virtual machine.

- Open PowerShell as Administrator.

- Run the following command:

wsl --install

- Restart your device when prompted

## 2.2 Installing Ubuntu Linux Distribution

After WSL is installed, set up an Ubuntu distribution:

- Open PowerShell as Administrator.

- Run the following command:

wsl --install Ubuntu

- Restart your device again once installation completes.

- When Ubuntu first launches, you will be prompted to create a Unix username and password. Choose a memorable username, it does not need to match your Windows credentials.

## 2.3 Opening WSL (Ubuntu Terminal)

There are two ways to open your Ubuntu terminal:

### Option A: Via the Windows Terminal UI

- Open Windows Terminal.

- Click the down arrow (⌄) next to the tab bar.

- Select Ubuntu from the list.

### Option B: Via Command

- Open Windows Terminal or PowerShell.

- Run:

wsl -d Ubuntu

# 3.0 Installing OpenFOAM

All commands in this section must be run inside the Ubuntu terminal. Run them in the exact order shown.

## Step 1: Add the OpenFOAM Debian repository

curl -s https://dl.openfoam.com/add-debian-repo.sh | sudo bash

## Step 2: Update the package list

sudo apt-get update

sudo apt-get upgrade

## Step 3: Install OpenFOAM

sudo apt-get install openfoam2506-default

Note: The installation may take several minutes depending on your internet connection. Accept any prompts with Y when asked.

# 4.0 Placing the Project Files

Create the OpenFOAM utilities directory and place the project scripts inside it.

## Step 1: Navigate to the root directory

Cd/

## Step 2: Create the directory

mkdir -p /mnt/c/OpenFOAM

mkdir 01_utilities

## Step 3: Place the scripts

Clone or manually copy the repository files so the directory contains at minimum:

- openfoam_ui.py               ← GUI entry point

- ui_background_mesh.py

- ui_snappy_hex.py

- ui_log_drawer.py

- ui_landing.py

- ui_shared.py

- setup_snappy.py

- encoding_utils.py

- auto_refinement.py

- generateBackgroundMesh.py    ← standalone CLI

- generateSnappyHexMeshDict.py ← standalone CLI

- defaults.json

- requirements.txt

- templates\

  - blockMeshDict.template

  - snappyHexMeshDict.template

- OpenFOAM_UI.exe             ← Windows launcher

Note: The path /mnt/c/OpenFOAM corresponds to C:\OpenFOAM on your Windows file system.

Note 2: Do NOT include __pycache__\ — it is machine-specific and regenerates automatically.

OR you can manually download the ZIP file then export it into the OpenFOAM folder you have created.

# 5.0 Installing Python and Python Dependencies

The mesh utility scripts require Python 3 along with several system packages. Run the following inside the Ubuntu terminal:

sudo apt-get install python3

sudo apt-get install python3-pip

sudo apt-get install python3-numpy python3-tk python3-jinja2

Or you can use an alternative to install the libraries using requirements.txt

pip3 install -r /mnt/c/OpenFOAM/01_utilities/requirements.txt --break-system-packages

To install any additional Python library in future:

sudo apt-get install python3-<library_name>

Note: Replace <library_name> with the actual package name, for example python3-scipy.

## 5.1 Packages Used

| Package | Required | Purpose |
| --- | --- | --- |
| PyQt5 | Yes | GUI framework |
| numpy | Yes | Bounding box arithmetic |
| jinja2 | Yes | Dictionary template rendering |
| trimesh | Optional | AUTO_ auto-refinement geometry analysis |

# 6.0 Creating Bash Aliases

Aliases let you run long commands with short, memorable names. Follow these steps inside the Ubuntu terminal.

## Step 1: Open the aliases file in vi

vi ~/.bash_aliases

## Step 2: Enter Insert mode

Press the i key to enter Insert mode. You will see -- INSERT -- at the bottom of the terminal.

## Step 3: Add the alias definitions

Type or paste the following content:

alias myDir="cd /mnt/c/OpenFOAM"

# OpenFOAM version

alias of2506="source /usr/lib/openfoam/openfoam2506/etc/bashrc"

# Python scripts for OpenFOAM workflow

alias generateBackgroundMesh="python3 /mnt/c/OpenFOAM/01_utilities/generateBackgroundMesh.py"

alias generateSnappyHexMeshDict="python3 /mnt/c/OpenFOAM/01_utilities/generateSnappyHexMeshDict.py"

alias openfoamUI="python3 /mnt/c/OpenFOAM/01_utilities/openfoam_ui.py"

alias setup_snappy="python3 /mnt/c/OpenFOAM/13_snappy_package_malaysian_team/demo_case_2/setup_snappy.py"

## Step 4: Save and exit

- Press Esc to exit Insert mode.

- Type command below to Save and Quit.

:wq

and press Enter to save and quit.

Quick vi command reference:

| Command | Action |
| --- | --- |
| :q | Quit without saving |
| :q! | Force quit and discard changes |
| :x | Save and quit (only if changes were made) |
| i | Enter Insert mode |
| Esc | Exit Insert mode |

## Step 5: Apply the aliases to your current session

source ~/.bash_aliases

Note: You only need to run source once per terminal session. New terminal sessions will load the aliases automatically.

# 7.0 Launching OpenFOAM

Every time you start a new Ubuntu terminal session, follow these two steps before running any OpenFOAM command or utility:

## Step 1: Source the OpenFOAM environment

of2506

## Step 2: Navigate to the working directory

myDir

Note: You must source of2506 in every new terminal session. OpenFOAM commands such as blockMesh and snappyHexMesh will not be available until you do.

# 8.0 Using the GUI

Once the OpenFOAM environment is sourced and you are in the working directory, you can run the utilities using the aliases defined in Section 7.

## 8.1 Graphical User Interface (Recommended)

The GUI wraps both utilities in a single window with form inputs and a live output log.

openfoamUI

Or double-click OpenFOAM_UI.exe in the 01_utilities\ folder. The startup window runs environment checks and opens the GUI automatically.

## 8.2 Landing Page

On launch the GUI shows a landing page where you:

- New project — enter a name and location; the tool creates the folder structure (constant/triSurface/, system/, 0/) and stub dictionaries (controlDict, fvSchemes, fvSolution).

- Open existing — browse to or pick from the recent-projects list (max 10; each has a × button to remove it). Validates that system/controlDict exists.

Choose a utility (Background Mesh or SnappyHexMesh Dict) and click Continue →. The ← Home button in the header bar returns to the landing page at any time.

Note: The working directory does not need to be set before launching — the landing page handles it.

## 8.3 Tab 1: Background Mesh

Generates system/blockMeshDict from an STL bounding box and runs blockMesh.

- STL file — browse or paste path; auto-detects the case root from constant/ in the path.

- Grid resolution — DX / DY / DZ cell sizes in metres.

- Click Generate Background Mesh — runs surfaceCheck, writes blockMeshDict, runs blockMesh, creates <case>.foam.

- Cancel — stops a running job and clears all input fields.

## 8.4 Tab 2: SnappyHexMesh Dict

Five-section card form that writes system/snappyHexMeshDict via Jinja2 template rendering and optionally runs snappyHexMesh.

| Section | Content |
| --- | --- |
| 01 Geometry | File table listing all STL/OBJ files found under constant/ (any subfolder); set Surface Type, min/max refinement levels, and Volume Direction + level per file |
| 02 Castellation | Geometry unit, nCellsBetweenLevels, location-in-mesh X Y Z |
| 03 Snap Controls | Implicit feature snapping toggle |
| 04 Layer Addition | Enable boundary layers; per-patch nSurfaceLayers spinboxes |
| 05 Generate & Run | - Generate snappyHexMeshDict merges GUI values with defaults.json and renders system/snappyHexMeshDict (plus fvSchemes/fvSolution when layers are on) - Run snappyHexMesh streams the solver to the log and refreshes the .foam file |

## 8.5 Output Log

The Output Log at the bottom of the window starts expanded and streams all subprocess output with colour-coded tags (error → red, warn → amber, info → blue, cmd → grey). Drag its bottom grip upward to resize it, or click the chevron to collapse/expand.

# 9.0 Using the CLI Scripts

## 9.1 Generate Background Mesh

Navigate to your OpenFOAM case directory first, then run:

cd /path/to/your/case

generateBackgroundMesh -stlPath /path/to/geometry.stl -dx 0.05 -dy 0.05 -dz 0.05

- Arguments:

- -stlPath   Path to the STL geometry file.

- -dx        Base grid size in the X direction (positive number).

- -dy        Base grid size in the Y direction (positive number).

- -dz        Base grid size in the Z direction (positive number).

## 9.2 Generate snappyHexMeshDict

Navigate to your OpenFOAM case directory first, then run:

cd /path/to/your/case

generateSnappyHexMeshDict

The script will prompt you interactively for geometry files, refinement levels, snap settings, and layer addition options.

## 10.0 Case Directory Requirements

A valid working directory must follow the following structure

- my_case

  - constant

    - triSurface

      - Fan.stl

      - Box.stl

      - Gear.stl

  - system

    - controlDict

    - fvSchemes

    - fvSolution

The GUI scans all constant/ recursively for .stl and .obj files. The subfolder name does not matter.

To use your own case:

- Ensure the case has both constant/ and system/ folders.

- Place STL/OBJ geometry files inside any subfolder of constant/.

- Use the landing page to browse to or create the case.

# 11.0 Prerequisites Summary

| Requirement | Notes |
| --- | --- |
| Windows 10 Build 21362 or Windows 11 | Required for WSLg (GUI apps from WSL) |
| WSL2 | wsl --install sets this up; wsl --status shows current version |
| OpenFOAM v2506 inside WSL | May refer to the OpenFOAM installation section |
| Python 3 + pip inside WSL | Usually pre-installed on Ubuntu; check with python3 --version |
| PyQt5, numpy, jinja2 inside WSL | All the python libraries in the |
| trimesh inside WSL | Optional — only needed for AUTO_ auto-refinement filenames |

# 12.0 What the Laucher Checks

The startup window runs six checks in order and stops at the first failure with a clear error message and fix instructions.

| No. | Check | If Missing |
| --- | --- | --- |
| 1 | WSL2 reachable | Install WSL2: wsl --install (PowerShell as Admin), restart Windows |
| 2 | WSLg display | wsl --update then wsl --shutdown |
| 3 | OpenFOAM 2506 | Install OpenFOAM 2506 in WSL — see Steps 1–4 above |
| 4 | Python 3 | sudo apt-get install -y python3 python3-pip inside WSL |
| 5 | Python packages | Launcher offers auto-install; or user can manually install as mentioned in the Python libraries and dependencies installation |
| 6 | openfoam_ui.py present | Keep all files in the same 01_utilities\ folder. Keep .exe |

# 13.0 Troubleshooting

## 13.1 Launcher Error Message

WSL Not Found

wsl.exe is not installed on this machine.

Fix:

Open PowerShell as Administrator and run: wsl --install Restart Windows, then try the launcher again. Contact IT if WSL is blocked by policy.

WSL Timed Out

WSL did not respond within 10 seconds.

Fix:

wsl --shutdown Wait 10 seconds, then run the launcher again.

WSL Unreachable

WSL responded with an unexpected exit code.

Fix:

wsl --status wsl --shutdown Then try again. If the error persists, run wsl --update and restart.

No Display Available

$DISPLAY and $WAYLAND_DISPLAY are both unset — WSLg is not active.

Fix:

wsl --update wsl --shutdown WSLg requires Windows 10 Build 21362+ or Windows 11 with WSL2.

Wrong OpenFOAM Version Installed

OpenFOAM v2506 is required.

Fix:

Install OpenFOAM 2506 inside WSL. Follow the Debian/Ubuntu precompiled package guide at https://develop.openfoam.com/Development/openfoam

OpenFOAM Not Found

No supported OpenFOAM bashrc was found.

Fix:

Install OpenFOAM 2506 inside your WSL Ubuntu environment

Python 3 Not Found in WSL

python3 is not available inside WSL.

Fix:

sudo apt-get update sudo apt-get install -y python3 python3-pip

Missing Python Packages

One or more of PyQt5, numpy, jinja2, trimesh are missing.

Fix:

sudo apt-get install -y python3-pyqt5 python3-numpy python3-jinja2

Package Installation Failed

pip3 install ran but returned an error.

Fix:

Open a WSL terminal and run the install command manually to see the full error. Common causes: no internet access in WSL, or a corporate proxy blocking pip.

Application File Missing

openfoam_ui.py is not in the same folder as OpenFOAM_UI.exe.

Fix:

Keep the .exe inside the 01_utilities folder alongside all the .py files. Do not distribute the .exe alone.

Launch Failed

wsl.exe was found but the process could not be started.

Fix:

Open a WSL terminal and launch manually:

source /usr/lib/openfoam/openfoam2506/etc/bashrc python3

/mnt/c/OpenFOAM/01_utilities/openfoam_ui.py

Check terminal output for Python errors.

## 13.2 General Issues

| Symptom | Resolution |
| --- | --- |
| OpenFOAM commands not found (e.g. blockMesh: command not found) | You have not sourced the environment. Run: of2506 |
| wsl --install fails or WSL version is 1 | Enable the 'Virtual Machine Platform' Windows feature and update WSL: wsl --update |
| python3: command not found | sudo apt-get install python3 |
| No module named 'PyQt5' | sudo apt-get install python3-pyqt5 |
| No module named 'jinja2' | sudo apt-get install python3-jinja2 |
| blockMesh: command not found | Source the OpenFOAM environment first |
| Blank window / no display | Windows 10: start VcXsrv or MobaXterm; Windows 11: WSLg should work out of the box |
| Not found: .../constant | The selected directory is not an OpenFOAM case root — it must contain constant/ and system/ |
| No files appear in Tab 2 geometry table | No .stl or .obj files found under constant/; check file placement |
| ParaView button does nothing | Install ParaView on Windows under C:\Program Files\ParaView*\ |
| Could not parse stylesheet in terminal | Harmless Qt5 warning on Linux/WSL — the GUI suppresses these automatically; no action needed |